# Документация по использованию Linux

Пошаговый план по этапам

Команды приведены в том виде, как они записаны в исходной документации. Спорные места собраны в разделе «Замечания» в конце файла.

# Модуль 1. Изучение сети

## Этап 1. Настройка hostname

Устройства: все устройства

### ISP

hostnamectl hostname isp

exec bash

### HQ-RTR (EcoRouter)

en

conf t

hostname hq-rtr.au-team.irpo

### BR-RTR (EcoRouter)

en

conf t

hostname br-rtr.au-team.irpo

### HQ-SRV

hostnamectl hostname hq-srv.au-team.irpo

exec bash

### BR-SRV

hostnamectl hostname br-srv.au-team.irpo

exec bash

### CLI

hostnamectl hostname cli.au-team.irpo

exec bash

## Этап 2. Настройка сети и NAT

Устройства: ISP, HQ-RTR, BR-RTR, HQ-SRV, BR-SRV

### ISP

### Копирование интерфейсов

cd /etc/net/ifaces

cp -r ens18 /etc/net/ifaces/ens19

cp -r ens19 /etc/net/ifaces/ens20

### ens19: options и адрес

cd ens19

vim options

# BOOTPROTO=static

echo 172.16.4.1/28 > ipv4address

### ens20: options и адрес

cd /etc/net/ifaces/ens20

vim options

# BOOTPROTO=static

echo 172.16.5.1/28 > ipv4address

### Включение forwarding

vim /etc/net/sysctl.conf

# net.ipv4.ip_forward = 1

### NAT (iptables) и проверка

apt-get update

apt-get -y install iptables

iptables -t nat -A POSTROUTING -o ens18 -j MASQUERADE

iptables-save >> /etc/sysconfig/iptables

systemctl enable --now iptables

systemctl restart network

ping ya.ru

ping ya.ru -I 172.16.4.1

ping ya.ru -I 172.16.5.1

### HQ-RTR (EcoRouter): интерфейсы и сервисы

do sh port br

int isp

ip address 172.16.4.14/28

ex

int 100

ip address 192.168.1.1/26

ex

int 200

ip address 192.168.1.65/28

ex

int 999

ip address 192.168.1.81/29

ex

port te0

service-instance isp

encapsulation untagged

connect ip interface isp

ex

ex

port te1

service-instance 100

encapsulation dot1q 100

rewrite pop 1

connect ip interface 100

ex

service-instance 200

encapsulation dot1q 200

rewrite pop 1

connect ip interface 200

ex

service-instance 999

encapsulation dot1q 999

rewrite pop 1

connect ip interface 999

ex

ex

### HQ-RTR: маршрут, DNS и NAT

ip route 0.0.0.0/0 172.16.4.1

ip name-server 77.88.8.8

ip nat pool INTERNET 192.168.1.1-192.168.1.87

ip nat source dynamic inside-to-outside pool INTERNET overload 172.16.4.14

int isp

ip nat outside

ex

int 100

ip nat inside

ex

int 200

ip nat inside

ex

int 999

ip nat inside

ex

ex

wr

### BR-RTR (EcoRouter): интерфейсы и сервисы

do sh port br

int isp

ip address 172.16.5.14/28

ex

int lan

ip address 192.168.2.1/27

ex

port te0

service-instance isp

encapsulation untaged

connect ip interface isp

ex

ex

port te1

service-instance lan

encapsulation untaged

connect ip interface lan

ex

ex

### BR-RTR: маршрут, DNS и NAT

ip route 0.0.0.0/0 172.16.5.1

ip name-server 77.88.8.8

ip nat pool INTERNET 192.168.2.1-192.168.2.30

ip nat source dynamic inside-to-outside pool INTERNET overload 172.16.5.14

int isp

ip nat outside

ex

int lan

ip nat inside

ex

ex

wr

### HQ-SRV

cd /etc/net/ifaces/ens18

echo 192.168.1.2/26 > ipv4address

echo default via 192.168.1.1 > ipv4route

echo nameserver 77.88.8.8 > resolv.conf

systemctl restart network

### BR-SRV

cd /etc/net/ifaces/ens18

echo 192.168.2.2/26 > ipv4address

echo default via 192.168.2.1 > ipv4route

echo nameserver 77.88.8.8 > resolv.conf

systemctl restart network

## Этап 3. GRE-туннель

Устройства: HQ-RTR, BR-RTR

### BR-RTR

en

conf t

int tunnel.1

ip address 10.10.10.10/30

ip tunnel 172.16.5.14 172.16.4.14 mode gre

ex

ex

wr

### HQ-RTR

en

conf t

int tunnel.1

ip address 10.10.10.9/30

ip tunnel 172.16.4.14 172.16.5.14 mode gre

ex

ex

wr

## Этап 4. Настройка OSPF

Устройства: HQ-RTR, BR-RTR

### BR-RTR

en

conf t

router ospf 1

network 10.10.10.8/30 area 0

network 192.168.2.0/27 area 0

area 0 authentication message-digest

passive-interface isp

passive-interface lan

ex

int tunnel.1

ip ospf authentication-key P@ssw0rd

ip ospf authentication message-digest

ex

ex

wr

do sh ip route

### HQ-RTR

en

conf t

router ospf 1

network 10.10.10.8/30 area 0

network 192.168.1.0/26 area 0

network 192.168.1.64/28 area 0

network 192.168.1.80/29 area 0

passive-interface isp

passive-interface 100

passive-interface 200

passive-interface 999

area 0 authentication message-digest

ex

int tunnel.1

ip ospf authentication-key P@ssw0rd

ip ospf authentication message-digest

ex

ex

wr

do sh ip route

## Этап 5. Настройка DNS

Устройства: HQ-SRV, HQ-RTR, BR-RTR

### HQ-SRV: установка

apt-get update

apt-get install bind bind-utils -y

cd /etc/bind

В named.conf закомментировать строку #include "/etc/bind/rndc.conf"; (поставить # перед строкой).

### options.conf

listen-on { 192.168.1.2; };

//listen-on-v6 { ::1; };

forwaders { 77.88.8.8; };

allow-query { any; };

allow-query-cache { any; };

allow-recursion { any; };

### local.conf

zone "au-team.irpo" {

type master;

file "/etc/bind/zone/db.au";

};

zone "1.168.192.in-addr.arpa" {

type master;

file "/etc/bind/zone/db.revers";

};

### Создание файлов зон

cd zone

cp localhost db.au

mcedit db.au

# заменить localhost -> au-team.irpo, 127.0.0.1 -> 192.168.1.2

cp db.au db.revers

chown root:named db.*

### db.au: прямая зона (дописать)

hq-srv IN A 192.168.1.2

hq-rtr IN A 192.168.1.1

br-srv IN A 192.168.2.2

bt-rtr IN A 192.168.2.1

hq-cli IN A 192.168.1.66

moodle IN CNAME hq-rtr

wiki IN CNAME hq-rtr

### db.revers: обратная зона (дописать)

1 IN PTR hq-rtr.au-team.irpo.

2 IN PTR hq-srv.au-team.irpo.

66 IN PTR hq-cli.au-team.irpo.

### Проверка зон и запуск

named-checkzone au-team.irpo db.au

named-checkzone 1.168.192.in-addr.arpa db.revers

systemctl restart bind

### resolv.conf и проверки

cd /etc/net/iface/ens19

mcedit resolv.conf

# nameserver 192.168.1.2

# search au-team.irpo

systemctl restart network

systemctl restart bind

host hq-srv

host 192.168.1.2

host wiki

host ya.ru

### HQ-RTR

en

conf t

ip name-server 192.168.1.2

ip domain-name au-team.irpo

ip domain-lookup

no ip name-server 77.88.8.8

do wr

do ping hq-srv

### BR-RTR

en

conf t

ip name-server 192.168.1.2

ip domain-name au-team.irpo

ip domain-lookup

no ip name-server 77.88.8.8

do wr

do ping hq-srv

## Этап 6. Настройка DHCP

Устройства: HQ-RTR

en

conf t

ip pool hq 192.168.1.67-192.168.1.78

dhcp-server 1

static ip 192.168.1.66

client-id mac XX:XX:XX:XX:XX:XX

mask 255.255.255.240

gateway 192.168.1.65

dns 192.168.1.2

domain_search au-team.irpo

ex

pool hq 1

mask 255.255.255.240

gateway 192.168.165

dns 192.168.1.2

domain_search au-team.irpo

do wr

int lan

int 200

dhcp-server 1

do wr

## Этап 7. Пользователи и SSH (задания 7–8)

Устройства: HQ-SRV, BR-SRV, HQ-RTR, BR-RTR

### HQ-SRV и BR-SRV: пользователь

useradd sshuser -u 1010

passwd sshuser

# P@ssw0rd

usermod -aG wheel sshuser

### /etc/sudoers

sshuser ALL=(ALL) NOPASSWD:ALL

### HQ-RTR и BR-RTR: пользователь

username net_admin

password P@ssw0rd

role admin

### HQ-SRV и BR-SRV: /etc/openssh/sshd_config

Port 2024

AllowUsers sshuser

MaxAuthTries 2

Banner /etc/banner.net

### /etc/banner.net

----------------------

Authorized access only

----------------------

### Перезапуск службы

systemctl restart sshd.service

## Этап 8. Настройка часового пояса

Устройства: все устройства

### ISP, BR-SRV, HQ-SRV, CLI (Alt Linux)

timedatectl set-timezone Europe/Moscow

timedatectl status

### BR-RTR, HQ-RTR (EcoRouter)

ntp timezone utc+3

show ntp timezone

# Модуль 2. Службы в корпоративной сети

## Этап 9. Настройка NTP / Chrony

Устройства: ISP + клиенты

### ISP: сервер Chrony

control chrony server

sed -i 's/pool pool.ntp.org iburst/pool pool.ntp.org iburst prefer minstratum 4/' /etc/chrony.conf

sed -i 's/#local stratum 10/local stratum 5/' /etc/chrony.conf

systemctl restart chronyd

chronyc sources

### Клиенты

sed -i 's/pool pool.ntp.org iburst/server 172.16.1.1 iburst/' /etc/chrony.conf

systemctl restart chronyd

chronyc sources

## Этап 10. Nginx reverse proxy + Basic Auth

Устройства: ISP

### Установка

apt-get update

apt-get install nginx -y

### /etc/nginx/sites-available.d/r-proxy.conf

server {

listen 80;

server_name web.au-team.irpo;

location / {

proxy_pass http://172.16.1.10:8080;

proxy_set_header Host $host;

proxy_set_header X-Real-IP $remote_addr;

proxy_set_header X-Forwarded-For $proxy_add_x_forwarded_for;

proxy_set_header X-Forwarded-Proto $scheme;

auth_basic "Restricted Access";

auth_basic_user_file /etc/nginx/.htpasswd;

}

}

server {

listen 80;

server_name docker.au-team.irpo;

location / {

proxy_pass http://172.16.2.10:8080;

proxy_set_header Host $host;

proxy_set_header X-Real-IP $remote_addr;

proxy_set_header X-Forwarded-For $proxy_add_x_forwarded_for;

proxy_set_header X-Forwarded-Proto $scheme;

}

}

### Симлинк и запуск

ln -s /etc/nginx/sites-available.d/r-proxy.conf /etc/nginx/sites-enabled.d/

nginx -t

systemctl enable --now nginx

systemctl status nginx

### HTTP Basic Auth

apt-get install apache2-htpasswd -y

htpasswd -c /etc/nginx/.htpasswd WEB

cat /etc/nginx/.htpasswd

nginx -t

systemctl restart nginx

## Этап 11. Статическая трансляция портов

Устройства: HQ-RTR, BR-RTR

### HQ-RTR

nft add chain nat prerouting { type nat hook prerouting priority dstnat \; }

nft add rule nat prerouting iif "enp7s1" tcp dport 2026 dnat to 192.168.1.10

nft add rule nat prerouting iif "enp7s1" tcp dport 8080 dnat to 192.168.1.10:80

nft list ruleset

nft list ruleset > /etc/nftables/nftables.nft

systemctl restart nftables

nft list ruleset

### BR-RTR

nft add chain nat prerouting { type nat hook prerouting priority dstnat \; }

nft add rule nat prerouting iif "enp7s1" tcp dport { 8080, 2026 } dnat to 192.168.3.10

nft list ruleset

nft list ruleset > /etc/nftables/nftables.nft

systemctl restart nftables

nft list ruleset

## Этап 12. Samba DC

Устройства: BR-SRV

### Подготовка DNS и сети

echo "nameserver 192.168.1.10" >> /etc/net/ifaces/enp7s1/resolv.conf

systemctl restart network

cat /etc/resolv.conf

ping ya.ru -c 2

### Установка Samba DC

apt-get update

apt-get install -y task-samba-dc

rm -f /etc/samba/smb.conf

rm -rf {/var/lib/samba, /var/cache/samba}

mkdir -p /var/lib/samba/sysvol

### Развёртывание домена

samba-tool domain provision

### Подмена krb5.conf

mv /etc/krb5.conf /etc/krb5.conf.back

cp /var/lib/samba/private/krb5.conf /etc/krb5.conf

### Запуск службы

systemctl enable --now samba

systemctl status samba

samba-tool domain info 127.0.0.1

### Добавление DNS-записей

samba-tool dns add br-srv.au-team.irpo au-team.irpo hq-srv A 192.168.1.10 -U Administrator

samba-tool dns add br-srv.au-team.irpo au-team.irpo hq-rtr A 192.168.1.1 -U Administrator

samba-tool dns add br-srv.au-team.irpo au-team.irpo br-rtr A 192.168.3.1 -U Administrator

samba-tool dns add br-srv.au-team.irpo au-team.irpo web.au-team.irpo A 172.16.1.1 -U Administrator

samba-tool dns add br-srv.au-team.irpo au-team.irpo docker.au-team.irpo A 172.16.2.1 -U Administrator

samba-tool dns query br-srv.au-team.irpo au-team.irpo @ ALL -U administrator

### Перевод DNS на локальный Samba DNS

sed -i 's/nameserver 192.168.1.10/nameserver 127.0.0.1/' /etc/net/ifaces/enp7s1/resolv.conf

systemctl restart network

cat /etc/resolv.conf

### Создание групп и пользователей

samba-tool group add hq

for i in {1..5}; do samba-tool user add hquser$i P@ssw0rd; done

for i in {1..5}; do samba-tool group addmembers hq hquser$i; done

samba-tool group listmembers hq

## Этап 13. Ввод HQ-CLI в домен

Устройства: HQ-RTR, HQ-CLI

### HQ-RTR: настройка DNS

sed -i 's/192.168.1.10/192.168.3.10/' /etc/dnsmasq.conf

systemctl restart dnsmasq

cat /etc/dnsmasq.conf

### HQ-CLI: ограничение входа группой hq

control libnss-role

roleadd hq wheel

### Ограничение sudo (/etc/sudoers)

WHEEL_USERS ALL=(ALL:ALL) /usr/bin/cat, /usr/bin/grep, /usr/bin/id

### Проверка

sudo id

## Этап 14. Ansible

Устройства: BR-SRV

### Установка

apt-get install ansible sshpass -y

### /etc/ansible/ansible.cfg

[defaults]

host_key_checking = False

interpreter_python = /usr/bin/python3

### /etc/ansible/hosts

HQ-SRV ansible_user=user ansible_password=resu ansible_port=2026

HQ-RTR ansible_user=net_admin ansible_password=P@ssw0rd

BR-RTR ansible_user=net_admin ansible_password=P@ssw0rd

HQ-CLI ansible_user=user ansible_password=resu

### Проверка

ansible all -m ping

## Этап 15. Docker

Устройства: BR-SRV

### Установка и загрузка образов

apt-get install docker-engine docker-compose-v2 -y

systemctl enable --now docker.service

mount -o loop /dev/sr0 /mnt/ -v

ls -l /mnt/docker/

cat /mnt/docker/readme.txt

docker load < /mnt/docker/site_latest.tar

docker load < /mnt/docker/mariadb_latest.tar

docker image ls

### docker-compose.yml

services:

database:

container_name: db

image: mariadb:latest

restart: always

ports:

- "3306:3306"

environment:

MARIADB_DATABASE: testdb

MARIADB_USER: test

MARIADB_PASSWORD: P@ssw0rd

MARIADB_ROOT_PASSWORD: P@ssw0rd

volumes:

- db_data:/var/lib/mysql

app:

container_name: testapp

image: site:latest

restart: always

ports:

- "8080:8000"

environment:

DB_HOST: database

DB_PORT: 3306

DB_NAME: testdb

DB_USER: test

DB_PASS: P@ssw0rd

DB_TYPE: maria

depends_on:

- database

volumes:

db_data:

### Запуск

docker compose config

docker compose up -d

docker ps

ss -ltnp4 | grep 8080

Проверка с HQ-CLI: http://docker.au-team.irpo и http://192.168.3.10:8080

### Пересоздать стек (если нужно)

docker rm -f $(docker ps -qa)

docker compose up -d

## Этап 16. RAID0

Устройства: HQ-SRV

### Разметка дисков

lsblk

parted /dev/sdb

### В parted (для /dev/sdb и /dev/sdc)

mklabel msdos

mkpart primary 1MiB 100%

set 1 raid on

print

# затем для второго диска:

select /dev/sdc

### Создание массива

mdadm --create /dev/md0 --level=0 --raid-devices=2 /dev/sdb1 /dev/sdc1

mdadm --detail --scan >> /etc/mdadm.conf

mkfs.ext4 /dev/md0

mkdir /raid

cp /etc/fstab /etc/fstab.back

echo "/dev/md0 /raid ext4 defaults 0 0" >> /etc/fstab

mount -av

df -T

## Этап 17. NFS и автомонтирование

Устройства: HQ-SRV, HQ-CLI

### HQ-SRV

apt-get update

apt-get install nfs-server nfs-utils -y

mkdir /raid/nfs

chmod 777 /raid/nfs

cp /etc/exports /etc/exports.back

echo "/raid/nfs 192.168.2.0/27(rw,no_subtree_check,no_root_squash)" >> /etc/exports

systemctl enable --now nfs-server

### HQ-CLI

mkdir /mnt/nfs

chmod -R 777 /mnt/nfs

showmount -e hq-srv

cp /etc/fstab /etc/fstab.back

echo "192.168.1.10:/raid/nfs /mnt/nfs nfs rw,soft,_netdev 0 0" >> /etc/fstab

mount -av

df -T

Проверка: создать файл в /mnt/nfs и увидеть его на стороне сервера.

## Этап 18. Веб-приложение (LAMP)

Устройства: HQ-SRV, HQ-CLI

### Установка LAMP и копирование файлов

mount -o loop /dev/sr0 /mnt/ -v

apt-get install lamp-server -y

cp /mnt/web/index.php /var/www/html

cp /mnt/web/logo.png /var/www/html

systemctl enable --now mariadb

### БД и пользователь

mariadb -e "CREATE DATABASE webdb;"

mariadb -e "

CREATE USER 'web'@'localhost' IDENTIFIED BY 'P@ssw0rd';

GRANT ALL PRIVILEGES ON webdb.* TO 'web'@'localhost';

"

### Импорт дампа

mariadb webdb < /mnt/web/dump.sql

mariadb -e "USE webdb; SHOW TABLES;"

### index.php: параметры подключения

$servername = "localhost";

$username = "web";

$password = "P@ssw0rd";

$dbname = "webdb";

### Запуск веб-сервера

systemctl enable --now httpd2.service

Проверка на HQ-CLI: открыть web.au-team.irpo, авторизация WEB / P@ssw0rd

## Этап 19. Установка Яндекс Браузера

Устройства: HQ-CLI

apt-get update

apt-get install yandex-browser-stable -y

# Чек-лист перед сдачей

- Chrony на ISP работает

- Nginx reverse proxy и Basic Auth подняты

- NAT портов на HQ-RTR и BR-RTR настроен

- Samba DC на BR-SRV создан

- Пользователи hquser1..hquser5 добавлены в группу hq

- Ansible с BR-SRV отвечает pong

- Docker-стек на BR-SRV работает

- RAID0 и NFS на HQ-SRV настроены

- Веб-приложение на HQ-SRV запускается

- Яндекс Браузер установлен на HQ-CLI

# Замечания: что стоит проверить в исходнике

Команды выше скопированы без изменений. Ниже места, где, судя по тексту, возможны опечатки или несостыковки. Сверьте их с заданием.

### Опечатки

- Этап 2, BR-RTR: «encapsulation untaged» (в HQ-RTR написано untagged).

- Этап 5, options.conf: «forwaders» должно быть forwarders.

- Этап 5, db.au: «bt-rtr» должно быть br-rtr.

- Этап 5: путь /etc/net/iface/ens19 (в других местах /etc/net/ifaces/...).

- Этап 6: «gateway 192.168.165» должно быть 192.168.1.65.

- Этап 6: строка «int lan» перед «int 200» выглядит лишней для HQ-RTR.

- Этап 2, BR-SRV: маска /26, а в сети BR-RTR используется /27 (192.168.2.1/27).

### Расхождения в адресах и портах

- Адреса ISP в этапе 2 (172.16.4.1 и 172.16.5.1) не совпадают с адресами в Chrony (172.16.1.1) и Nginx (172.16.1.10, 172.16.2.10).

- Порт SSH на серверах 2024 (этап 7), а в пробросе портов и Ansible используется 2026.

- В этапах 11–13, 15, 17 используются адреса 192.168.1.10, 192.168.3.10, 192.168.3.1 и интерфейс enp7s1. В этапах 2–6 серверы имеют адреса 192.168.1.2 и 192.168.2.2 и интерфейс ens18.

- Этап 17: экспорт NFS открыт для сети 192.168.2.0/27 (сеть BR), хотя клиент HQ-CLI находится в 192.168.1.64/28.

- Этап 11: на HQ-RTR и BR-RTR (EcoRouter) используются команды nft, но в этапах 2–4 эти же устройства настраиваются через CLI EcoRouter.

- Обратная зона 1.168.192.in-addr.arpa (этап 5) охватывает только адреса 192.168.1.x. Записей для BR-сети в ней нет.

### Неполные места

- Этап 12: команда «samba-tool domain provision» запускается интерактивно, параметры (realm, domain, пароль) нужно ввести вручную.

- Этап 13: в разделе «Ввод HQ-CLI в домен» приведены только ограничение входа и sudo. Самой команды ввода в домен в исходнике нет.

- Этап 14: для HQ-SRV и HQ-CLI указан пользователь user с паролем resu, а в этапе 7 создаётся sshuser.